# Chapter 2

# Installation

## Introduction

This chapter describes the software and hardware required to use SNMPc, and the installation process.

## Requirements

The following hardware and software components must be installed before SNMPc. These components are not included with SNMPc and must be purchased separately.

1.	IBM PC/AT 486 or compatible with 8MB memory.

2.	Hard disk drive with at least 50 MB free space.

3.	3.5" high density diskette drive.

4	VGA color adapter and monitor.

5.	Mouse.

6.	MS-DOS version 3.0 or greater.

7.	Microsoft Windows 3.1 or greater.

8.	Network card. Please refer to protocol software documentation for a list of supported Network cards.

9.	Windows Sockets compliant TCP/IP stack.

10.	SNMP Agent software for each node.

## Software Copy Protection

SNMPc uses a serial number to protect against unauthorized duplication.  SNMPc checks the validity of the serial number each time it is started, and it also communicates with other copies of SNMPc running on your network.  If a duplicate serial number is detected, both copies of SNMPc will display a message and stop network access.

The serial number is printed on the License Agreement card.  You must not lose this card as it your proof of purchase.  You will be asked to enter the serial number when SNMPc is installed.   Note that TCP/IP stack you use may also have separate serial number and key which is used when installing the TCP/IP stack.

## Installing TCP/IP Software

SNMPc requires a Windows Sockets compliant TCP/IP stack.  Windows/95 and Windows/NT both include a free TCP/IP stack.   The TCP/IP protocol software must be installed and functional before SNMPc can be used.

You must assign an internet (IP) address for the PC, and for each node that will be managed by SNMPc. Refer to the protocol software documentation for instructions on how to set the IP addresses. Once you have completed this step, you should be able to use the protocol Ping command for the PC host name. You should also test the network interface configuration by Pinging another TCP/IP device that is connected to the network.

## Installing SNMPc

Make sure that the required components, which were listed earlier, are installed and tested. Once this is done, perform the following steps to install SNMPc:

1.	Start up Windows and place the SNMPc Program Disk 1 into a diskette drive.

2.	Select the File/Run command from the Program Manager.  Enter drive:\setup into the Command Line edit box of the Run Program dialog box. Substitute the letter of your diskette drive for drive.

3.	SNMPc setup will display an introduction screen. Press the Continue button to start the installation.  You may press the Exit button in the lower right of the screen at any time to abort the installation.

4.	SNMPc setup will ask for the user name, company name and SNMPc serial number in the next dialog box.  The serial number is printed on the SNMPc License Agreement card.

5.	SNMPc setup will now display the main installation dialog box.

Use the SNMPc Installation button to install the SNMPc platform, which includes the minimum required elements to use SNMPc (Program Disks 1 and 2).   If this is a new or SNMPc upgrade installation, you must use this button first.

Use the Vendor Installation button to install the Castle Rock Computing device support (Vendor Disks 1, 2, and 3) or a third party device support application.  You must install SNMPc using the SNMPc Installation button before installing any vendor disks.

Use the Exit Installation button to exit setup after installing SNMPc or Vendor support disks.

If You Select SNMPc Installation

1.	If you have previously installed SNMPc, setup will ask if you want to update the installed version.  If you answer no the following dialog box will prompt for the install directory. Enter the full path name and press OK.

2.	SNMPc setup will now install the required files.  A dialog box graphically displays the installation progress.  You will be prompted to insert Program Disk 2.  Insert the disk and press the OK button.

3.	After installing all files, SNMPc setup will search for an installed version of HP OpenView for Windows version 7.2.   If you have installed OpenView, SNMPc setup will ask if you would like to add SNMPc as a sub-application of HP OpenView.  If you answer yes SNMPc functionality will be available from the HP OpenView map and menus.

4.	SNMPc setup will create a new Windows program group, SNMPc 4.0, containing icons for the SNMPc main program, application and utility programs, and  a notepad readme release notice. SNMPc will also change the Windows files, win.ini and progman.ini, saving the original versions in win.bak and progman.bak.

5.	SNMPc setup will now return to the main installation dialog box.  Use the Vendor Installation button to install Castle Rock Computing or third party device support, or use the Exit Installation button to exit setup.

If You Select Vendor Installation

1.	Insert the SNMPc Vendor Disk 1 or the first disk of a third party device application in the diskette drive.  The following dialog box prompts for the required vendor support to install.  Select one or more entries in the vendor list box.  Use the Select All button to select all entries and the Deselect All button to select none.  Press OK to continue.

2.	SNMPc will install the requested vendor files.  A dialog box displays the installation progress.  You may be prompted to insert additional vendor disks.

3.	After SNMPc setup has installed the requested vendor support, the main installation dialog box will be displayed.  Use the Vendor Installation button to install additional vendor support disks, or use the Exit Installation button to exit setup.

## For More Information

Chapter 3, User's Guide, provides a tutorial explanation of how to use SNMPc.

Chapter 4, Command Reference, describes each SNMPc command in detail.