Remote Beeper Alarm System

The SNMPc Remote Beeper Alarm System is a program that will alert you of an error detected on your network via your beeper.  When used in conjunction with remote voice information system, SNMPc can tell you what has happened to your network when you call back.

Installation

installation of the SNMPc remote beeper alert program is done automatically when SNMPc itself is installed.  The files used by this program are BEEPER.EXE, the executable that is stored in the APIEXEC\ subdirectory, BEEPER.DAT which contains the information that is used by the program and is also in the APIEXEC\ subdirectory, and BEEPER.LOG which is created in your SNMPc directory to log errors that were associated with a beeper alert.  This program also requires a Hayes compatible modem that is used to dial the beeper.

Configuration

When the SNMPc remote beeper alert program is run without arguements, it runs the configuration portion of the program.  If the configuration file (APIEXEC\BEEPER.DAT) does not exist, you are alerted to this problem and the configuration program comes up.  The format of the BEEPER.DAT file is a follows:

[COM?] [Phone Number] [Beeper Number] [Wait (in seconds)]

Fields

Phone Number:	The phone number of your beeper service.

Beeper Number:	The number you want to appear on your beeper.

Wait:		Number of seconds to pause between dialing the phone

number and the entering of the beeper number.

COM1..4:	Serial port that your modem is connected to.

Butons

Ok:		Quit and save information.

Cancel:		Quit without saving information.

Defaults:	Reload last saved information.

Dial:		Test configuration.  Dials numbers to test if they are

correct and allows you to see if your Wait seconds are

correct.

Usage

To inform SNMPc to use the remote beeper alert, run SNMPc and go to the Config/Event Actions menu option.  When you edit an Even Action, there is a check box for API Exec and an edit box to enter the filename of the API to run.  Enter the filter information you want for a beeper alert then check this box and enter "BEEPER.EXE" in the edit field.  When the filter is detected, the remote beeper alarm will check to see if the file BEEPER.LOG has been created.  If not, it will create it and alert you of this error through the beeper.  When you call back, the remote voice information system will read the errors logged to the BEEPER.LOG file.